MULTIMEDIA UNIVERSITY OF KENYA

FACULTY OF SCIENCE AND TECHNOLOGY

DEPARTMENT OF  PHYSICS

INTRODUCTION TO COMMUNICATION ENGINEERING

CAT-1 DURATION  1HR

Attempt only three question

- (a) State the properties of signal as the function of communication system. Hence explain how signal system are classified.                                                                                           [5Marks]

(b) Why do we modulate our information? Explain briefly the basic function of switch square law modulator                                                                                                               [5Marks]

- If two musical programs were played at the same time within distance, it would be difficult for anyone to listen to one source and not hear the second source. Since all musical sounds have approximately the same frequency range, form about 50 Hz to 10KHz. If a desired program is shifted up to a band of frequencies between 100KHz and 110KHz, and the

- second program shifted up to the band between 120KHz and 130KHz, Then both programs gave still 10KHz bandwidth and the listener can (by band selection) retrieve the program of his own choice. The receiver would down shift only the selected band of frequencies to a suitable range of 50Hz to 10KHz.  frequency to be radiated. This is 75 meters at 1 MHz but at 15KHz it has increased to 5000 meters (or just over 16,000 feet) a vertical antenna of this size is impossible.

- The third reason for modulating a high frequency carrier is that RF (radio frequency)

energy will travel a great distance than the same amount of energy transmitted as sound  power. A second more technical reason to shift the message signal to a higher frequency is related to antenna size. It is to be noted that the antenna size is inversely proportional to the

- (a) Explain the functional block of analog communication model                              [2Marks]

- The transmitter itself is a collection of electronic components and circuits designed to convert the electrical signal to a signal suitable for transmission over a given communication medium. Transmitters are made up of oscillators, amplifiers, tuned circuits and filters, modulators, frequency mixers, frequency synthesizers, and other circuits.

- The communication channel is the medium by which the electronic signal is sent from one place to another. Many different types of media are used in communication systems, including wire conductors, fiber-optic cable, and free space.

- A receiver is a collection of electronic components and circuits that accepts the

- transmitted message from the channel and converts it back to a form understandable by humans. Receivers contain amplifiers, oscillators, mixers, tuned circuits and fillters, and a demodulator or detector that recovers the original intelligence signal from the modulated carrier.

(b)Discuss various transmission media used in the conventional communication model, states their advantages and disadvantages                                                                               [2Marks]

- Electrical Conductors. In its simplest form, the medium may simply be a pair of

wires that carry a voice signal from a microphone to a headset. It may be a coaxial cable such as that used to carry cable TV signals. Or it may be a twisted-pair cable used in a local-area network (LAN).

- Optical Media. The communication medium may also be a fiber-optic cable or “light

pipe” that carries the message on a light wave. These are widely used today to carry

long-distance calls and all Internet communications. The information is converted to digital form that can be used to turn a laser diode off and on at high speeds. Alternatively, audio or video analog signals can be used to vary the amplitude of the light.

- Free Space. When free space is the medium, the resulting system is known as radio.

so known as wireless, radio is the broad general term applied to any form of wireless

communication from one point to another. Radio makes use of the electromagnetic spectrum. Intelligence signals are converted to electric and magnetic fields that propagate nearly instantaneously through space over long distances. Communication by visible or infrared light also occurs in free space.

- Other Types of Media. Although the most widely used media are conducting cables

and free space (radio), other types of media are used in special communication systems. For example, in sonar, water is used as the medium. Passive sonar “listens” for underwater sounds with sensitive hydrophones. Active sonar uses an echo-reflecting technique similar to that used in radar for determining how far away objects under water are and in what direction they are moving.

(c) Deduce an expression for AM voltage and power distribution hence determine

[6Marks]

i. modulation index

ii. Sideband frequencies

iii. Amplitude of each side band frequencies

iv. Bandwidth

v. Total power delivered to the load of 1000Ω

when the audio frequency of 100cos 2ᴫ×1000t  is used to amplitude  a modulated carrier of 200sin 2ᴫ× 106 t

- Illustrate the functional block of superheterodyne receiver hence describe each unit and its designing parameters                                                                                                    [10Marks]

Front end amplifier and tuning block: Signals enter the front end circuitry from the

antenna. This circuit block performs two main functions:

o Tuning: Broadband tuning is applied to the RF stage. The purpose of this is to

reject the signals on the image frequency and accept those on the wanted

frequency. It must also be able to track the local oscillator so that as the receiver is

tuned, so the RF tuning remains on the required frequency. Typically the selectivity

provided at this stage is not high. Its main purpose is to reject signals on the image

frequency which is at a frequency equal to twice that of the IF away from the

wanted frequency. As the tuning within this block provides all the rejection for the

image response, it must be at a sufficiently sharp to reduce the image to an

acceptable level. However, the RF tuning may also help in preventing strong off channel

signals from entering the receiver and overloading elements of the

receiver, in particular the mixer or possibly even the RF amplifier.

o Amplification: In terms of amplification, the level is carefully chosen so that it

does not overload the mixer when strong signals are present, but enables the signals to be amplified sufficiently to ensure a good signal to noise ratio is achieved. The amplifier must also be a low noise design. Any noise introduced in this block will  be amplified later in the receiver.

Mixer / frequency translator block: The tuned and amplified signal then enters one port

of the mixer. The local oscillator signal enters the other port. The performance of the

mixer is crucial to many elements of the overall receiver performance. It should eb as

linear as possible. If not, then spurious signals will be generated and these may appear as

'phantom' received signals.

Local oscillator: The local oscillator may consist of a variable frequency oscillator that

can be tuned by altering the setting on a variable capacitor. Alternatively it may be a

frequency synthesizer that will enable greater levels of stability and setting accuracy.

Intermediate frequency amplifier, IF block : Once the signals leave the mixer they

enter the IF stages. These stages contain most of the amplification in the receiver as well

as the filtering that enables signals on one frequency to be separated from those on the

next. Filters may consist simply of LC tuned transformers providing inter-stage coupling,

or they may be much higher performance ceramic or even crystal filters, dependent upon

what is required.

Detector / demodulator stage: Once the signals have passed through the IF stages of the

superheterodyne receiver, they need to be demodulated. Different demodulators are

required for different types of transmission, and as a result some receivers may have a

variety of demodulators that can be switched in to accommodate the different types of

transmission that are to be encountered. Different demodulators used may include:

o AM diode detector: This is the most basic form of detector and this circuit block

would simple consist of a diode and possibly a small capacitor to remove any

remaining RF. The detector is cheap and its performance is adequate, requiring a

sufficient voltage to overcome the diode forward drop. It is also not particularly

linear, and finally it is subject to the effects of selective fading that can be apparent,

especially on the HF bands.

Selectivity

- Used to measure the ability of the receiver to accept a given band of frequencies and reject all others.

- Way to describe selectivity is to simply give the bandwidth of the receiver at the -3dB points.

- Not necessarily a good means of determining how well the receiver will reject unwanted frequencies.

- Bandwidth improvement

- Thermal noise directly proportional to bandwidth.

- Reduce BW ~ reduce noise, improving system performance.

- Reducing BW = improving the noise figure of the RX

- Bandwidth Improvement, BI

BI = BRF /BIF

Where BRF = RF Bandwidth (Hz)

BIF = IF Bandwidth (Hz)

Noise figure improvement,

NF = 10 log BI

- Sensitivity

- The minimum RF signal level that can be detected at the input to the Rx and still produce a usable demodulated information signal.

- Usually stated in micro volts of received signal.

- Rx sensitivity also called Rx threshold

Depends on:

- The noise power present at the input to the Rx.

- Rx noise figure.

- AM detector sensitivity.

- BI factor of the Rx

- To improve ~ reduce the noise level

- Reducing the temperature or Rx BW or RX noise figure

- Dynamic range

- The difference (in dB) between the minimum input level necessary to discern a signal and the input level that will overdrive the Rx and produce distortion.

- Input power range over which the Rx is useful.

- A dynamic range of 100dB is considered about the highest possible.

- A low dynamic range can cause a desensitizing of the RF amplifiers and result in severe intermodulation distortion of the weaker input signal

- Fidelity

- A measure of the ability of a communication system to produce (at the output of the Rx) an exact replica of the original source information.

  - Forms of distortion that can deteriorate the fidelity of a communication system:-

- Amplitude

- Frequency

- Phase

- Insertion Loss

- IL is a parameter associated with the frequencies that fall within the passband of a filter.

- The ratio of the power transferred to a load with a filter in the circuit to the power transferred to a load without the filter.

IL (dB) = 10 log (Pout /Pin)

Noise temperature & Equivalent noise temperature

- Thermal noise directly proportional to temperature ~ can be expressed in degrees, watts or volts.

- Environmental temperature, T (kelvin)

T = N/KB

Where  N = noise power (watts)

K = Boltzman’s Constant

(1.38 X 10-23 J/K)

B = Bandwidth (Hz)

- With aid of the block diagram, explain the detection of AM wave using a) square law detector b) envelope detector                                                                                  [10Marks]

A Square-law modulator requires nonlinear element and a low pass filter for  extracting the desired message signal. Semi-conductor diodes and transistors  are the most common nonlinear devices used for implementing square law  modulators. The filtering requirement is usually satisfied by using a single or  double tuned filters.

When a nonlinear element such as a diode is suitably biased and operated in a  restricted portion of its characteristic curve, that is ,the signal applied to the  diode is relatively weak, we find that transfer characteristic of diode-load  resistor combination can be represented closely by a square law :

V0 (t) = a1Vi (t) + a2 Vi2 (t) ……………….(i)  Where a1, a2 are constants

Now, the input voltage Vi (t) is the sum of both carrier and message signals  i.e., Vi (t) = Ac [1+kam (t)] cos2pfct …………….(ii)

Substitute equation (ii) in equation (i) we get

V0 (t) = a1Ac [1+kam (t)] cos2pfct +

1/2 a2Ac2 [1+2 kam (t) + ka2 m2 (t)] [cos4pfct]………..(iii)

Now design the low pass filter with cutoff frequency f is equal to the required  message signal bandwidth. We can remove the unwanted terms by passing  this output voltage V0 (t) through the low pass filter and finally we will get  required message signal.

V0 (t) = Ac2 a2 m (t)

The Fourier transform of output voltage VO (t) is given by

VO (f) = Ac2 a2 M (f)

M (f)

Ac2 a2 M(0)

Fig: Envelope detector

Envelope detector is used to detect high level modulated levels, whereas  square-law detector is used to detect low level modulated signals (i.e., below  1v). It is also based on the switching action or switching characteristics of a  diode. It consists of a diode and a resistor-capacitor filter.

The operation of the envelope detector is as follows. On a positive half cycle  of the input signal, the diode is forward biased and the capacitor C charges up  rapidly to the peak value of the input signal. When the input signal falls below  this value, the diode becomes reverse biased and the capacitor C discharges  slowly through the load resistor Rl . The discharging process continues until  the next positive half cycle. When the input signal becomes greater than the  voltage across the capacitor, the diode conducts again and the process is  repeated.

The charging time constant RsC is very small when compared to the  carrier period 1/fc i.e.,

RsC << 1/fc

Where Rs = internal resistance of the voltage source.

C = capacitor

fc = carrier frequency

i.e., the capacitor C charges rapidly to the peak value of the signal.

The discharging time constant RlC is very large when compared to the  charging time constant i.e.,

1/fc << RlC << 1/W

Where Rl = load resistance value

W = message signal bandwidth  i.e., the capacitor discharges slowly through the load resistor.

Advantages:

  - It is very simple to design

  - It is inexpensive

  - Efficiency is very high when compared to Square Law detector

Disadvantage:

    - Due to large time constant, some distortion occurs which is known  as diagonal clipping i.e., selection of time constant is somewhat  difficult

Application:

It is most commonly used in almost all commercial AM Radio  receivers.

- Discuss the advantages of SSB and its demerits hence explain the parametric that are used to measure the capability of the AM receiver.                                                                [10Marks]

SSB signals offer four major benefits

1. The primary benefits t of an SSB signal is that the spectrum space it occupies is only

one-half that of AM and DSB signals. This greatly conserves spectrum space and

allows more signals to be transmitted in the same frequency range.

2. All the power previously devoted to the carrier and the other sideband can be channeled

into the single sideband, producing a stronger signal that should carry farther

and be more reliably received at greater distances. Alternatively, SSB transmitters

can be made smaller and lighter than an equivalent AM or DSB transmitter because

less circuitry and power are used.

3. Because SSB signals occupy a narrower bandwidth, the amount of noise in the

signal is reduced.

4. There is less selective fading of an SSB signal over long distances. An AM signal is really multiple signals, at least a carrier and two sidebands. These are on different frequencies, so they are affected in slightly different ways by the ionosphere and upper atmosphere, which have a great influence on radio signals of less than about 50 MHz. The carrier and sidebands may arrive at the receiver at slightly different times, causing a phase shift that can, in turn, cause them to add in such a way as to cancel one another rather than add up to the original AM signal. Such cancellation, or selective fading, is not a problem with SSB since only one sideband is being transmitted.

- Selectivity

- Bandwidth improvement

- Sensitivity

- Dynamic range

- Fidelity

- Insertion Loss

- Noise temperature & Equivalent noise temperature

- (a)Why digital modulation and not analog modulation?Explain the relationship between QPSK and 4-QAM.                                                                                                     [10Marks]

Noise Immunity. When a signal is sent over a medium or channel, noise is invariably

added to the signal. The S/N ratio decreases, and the signal becomes harder to recover.

Noise, which is a voltage of randomly varying amplitude and frequency, easily corrupts

analog signals. Signals of insufficient amplitude can be completely obliterated by noise.

Error Detection and Correction. With digital communication, transmission errors

can usually be detected and even corrected. If an error occurs because of a very high

noise level, it can be detected by special circuitry

Compatibility with Time-Division Multiplexing. Digital data communication is

adaptable to time-division multiplexing schemes. Multiplexing is the process of transmitting

two or more signals simultaneously on a single communication channel or medium.

Digital ICs. A further benefit of digital techniques is that digital ICs are smaller and

easier to make than linear ICs, therefore can be more complex and provide a processing

capability greater than what can be accomplished with analog ICs.

Digital Signal Processing (DSP). DSP is the processing of analog signals by

digital methods. This involves converting an analog signal to digital and then processing

with a fast-digital computer. Processing means filtering, equalization, phase shifting,

mixing, and other traditionally analog methods

- Using illustration, explain the both principle of BPSK, FSK on transmitter and receiver.

[10Marks]

- Explain the functional block of GSM architecture network;Enlist the characteristic of 3G and 4G.As the nation, what are the key challenges of rolling 4G networks. Discuss.        [10Marks]